Material Safety Data Sheet
Synthetic Iron Oxide: Yellow PA200, PA250, A70, A55, A50
Product / Material:

Synthetic Iron Oxide: Yellow 

Manufacturer: 

Cía. PILAR AQUAREL ARG. 






s.r.l.

ADDRESS:
Ruta 8 km. 48,500 esq. Güemes- La Lonja- Pilar

Section I –Product Identification






Trade Name: 



Red Iron Oxide

Synonym:



Iron Oxide, Ferric Oxide

Chemical Family:


Metal Oxide

Formula: 




Fe O.OH
Section II –Hazardous Ingredients
None

Section III –Physical Data
Boiling Point (º F):


Not Applicable 

Vapor Pressure (mm Hg):
Not Applicable

Vapor Density:


Not Applicable

Solubility in Water:

Insoluble

Specific Gravity (H2O=1):
3.6 / 4.2
Percent Volatile by Weight: 
0

Evaporation Rate: 


0

Appearance and Odor:

Powder, no Odor

SECTION IV –Fire and Explosion Hazard Data
Flash Point (Method Used):
Non-flammable
Flammable limits:
LEL

Non-flammable





UEL

Non-flammable

Extinguishing media:

Water

Section V –Health Hazard Data 

Threshold Limit Values: 
Inert or Nulsance dust








5 mg/M³ respirable (ACGIH & OSHA)








10 mg/M³ total dust ( ACGHI)








15 mg/M³ total dust (OSHA) 

Effects of Overexposure:
May cause mechanical irritation to eyes and respiratory tract.

Emergency and First aid 
Procedures:
Wash dust for skin with soap and water. Flush out eyes with generous amounts of water for at least 15 minutes. See a Physician.

Section VI –Reactivity Data

Product is stable
Incompatibility: None 

Hazardous Decomposition Products: None 

Hazardous Polymerization will not occur.

Section VII –Spill or Leak Procedures 
Steps to be taken in case 
material is realised or spilled:Normal clean-up procedures. Care Should be taken to avoid causing dust to become airborne. Vacuum cleaning systems are recommended.
Waste Disponsal Method:
Discarded iron would not be classified as a hazardous waste.

Section VIII –Special Protection Information

Eyes Protection (use most appropriate):Dust-tight goggles      in dusty environments. 
Skin Protection:

Use of dust respirator is recommended when exposure limits may be exceeded.

Ventilation:

Local exhaust ventilation to collector or containment recommended to control dust to bellow exposure limits.

Section IX –Special Precautions 
Good industrial hygiene practice requires that employee exposure be maintained below the recommended TVL. This preferably achieved through the provision of adequate ventilation where necessary. Where dust cannot be controlled in this way, personal respiratory protection should be employed.

Notice: The data and recommendation presented herein are based upon data which is considered to be. However, Cía. Pilar Aquarel Arg. S.r.l. grants no guarantee or warranty, either expressed or implied, of the accuracy or completeness of these data and recommendations.

